Title : Ultra-Low Power Smart Logger

Objective :-

Design an ultra-low power ESP32-based environmental logger that can operate in harsh remote conditions like forests, deserts, or glaciers for a minimum of 3 months on a single battery pack.

Sensors & Components :-

ESP32 Development Board - Main microcontroller with Wi-Fi, Bluetooth, deep           s                                                  sleep support

MicroSD Card Module - Local storage of logged data

BMP280 sensor - Digital temperature and barometric pressure sensor

PIR Sensor - 	Detects motion to wake up ESP32 from deep sleep

TP 4056 charging Module - Safe battery charging and protection circuit

Lithium ion Battery - Primary power source

Softwares used :-

Kicad - Schematic capture, PCB layout, and electronic design automation (EDA).

Blynk - IoT platform for remote monitoring, visualization, and basic device    m  a           management.

Architecture Diagram :-

TP 4056 charging module

Esp-32 Development Board

SD Card (local memory)

Blynk (cloud storage)

BMP280 sensor

PIR Sensor

Lithium ion Battery

Project Description :-

This project involves designing and building a low-power environmental data logger using the ESP32-DEVKITC-32D development board. The device is intended for remote or outdoor deployments where continuous monitoring of environmental parameters is required but stable power sources are unavailable. The device remains in a low-power deep sleep mode most of the time, waking up either based on a timer or when motion is detected by the PIR sensor. Upon waking, it reads sensor data, logs the data with timestamps on the SD card, uploads the current readings to the Blynk cloud for remote viewing, and runs OTA update checks. After completing its operations, it returns to deep sleep to conserve battery.

- Battery Capacity (C) = 1200 mAh

- Active Current (I_active) = 150 mA

- Active Time (t_active) = 15 seconds = 0.00417 hours

- Sleep Current (I_sleep) = 0.1 mA

- Sleep Time (t_sleep) = 6 – 0.00417 ≈ 5.9958 hours

Average Current Draw:

Iavg=Iactive×tactive+Isleep×tsleeptactive+tsleep=(150×0.00417)+(0.1×5.9958)6=0.6255+0.59966=1.22516=0.2042 mA

Estimated Battery Life:  Battery life (hours)=12000.2042≈5878 hours

Convert to days:     587824≈245 days≈8 months

Blynk (Cloud Interface):

Data Storage format in local storage  (SD card) :-

- Date: DD/MM/YYYY, Time: HH:MM:SS, Motion: 0/1,

Temperature: XX.XX celcius, pressure: XXXX.XX, voltage: XX.XXV

- Date: 27/07/2025, Time: 21:00:00, Motion: 0,

Temperature: 25.35 celcius, pressure: 1007.50, voltage: 3.78V

Budget sheet:

| component | Price | Product link |
| --- | --- | --- |
| SD card reader module | 39rs | https://robu.in/product/micro-sd-card-module/ |
| TP 4056 | 12rs | https://robu.in/product/tp4056-1a-li-ion-lithium-battery-charging-module-with-current-protection-type-c/ |
| PIR motion sensor | 82rs | https://robu.in/product/as312-senba-sensing-tech-plugin-human-body-sensing-sensor-rohs/ |
| ESP – 32 Development Board | 557rs | https://robu.in/product/esp32-wroom-32d-iot-development-doard-module-for-arduino/ |
| BMP 280 sensor | 25rs | https://robu.in/product/bmp280-barometric-pressure-and-altitude-sensor-i2c-spi-module/ |
| Lithium-Ion Rechargeable Battery | 35rs | https://makerbazar.in/products/18650-3-7v-lithium-ion-rechargeable-cell-good-quality?variant=46013701423344 |